Vehicle-to-Vehicle (V2V) communication protocol and collision detection system

🚗 Transmitter Node Connections

- Ultrasonic Sensor (HC-SR04)

  - TRIG → GPIO 5 (ESP32)

  - ECHO → GPIO 18 (ESP32)

- GPS Module

  - TX → GPIO 16 (ESP32, RX pin of HardwareSerial1)

  - RX → GPIO 17 (ESP32, TX pin of HardwareSerial1)

  - Baud rate: 9600

- ESP32 Board

  - Configured in WiFi Station Mode (WIFI_STA)

  - Sends data via ESP-NOW protocol

- Power Supply

  - 5V/3.3V regulated supply for ESP32 and sensors

🚙 Receiver Node Connections

- OLED Display (SSD1306, I2C)

  - SDA → GPIO 21 (ESP32 default I2C SDA)

  - SCL → GPIO 22 (ESP32 default I2C SCL)

  - Address: 0x3C

- Buzzer

  - Positive terminal → GPIO 4 (ESP32, OUTPUT)

  - Negative terminal → GND

- ESP32 Board

  - Configured in WiFi Station Mode (WIFI_STA)

  - Receives data via ESP-NOW callback function

- Power Supply

  - 5V/3.3V regulated supply for ESP32, OLED, and buzzer

🔄 Data Flow Connections

- Transmitter ESP32

  - Reads distance from ultrasonic sensor

  - Reads GPS data (lat, lon, speed)

  - Packages into Message struct

  - Sends via ESP-NOW to receiver MAC address

- Receiver ESP32

  - Receives packet via callback

  - Decodes struct (distance, speed, lat, lon)

  - Displays values on OLED

  - Activates buzzer if:

    - Distance < 20 cm OR Speed > 60 km/h

⚡ Summary of Key Wiring

- Transmitter: Ultrasonic (GPIO 5, 18), GPS (GPIO 16, 17)

- Receiver: OLED (GPIO 21, 22), Buzzer (GPIO 4)

TRANSMISSION NODE

RECEIVER NODE